Instructions for Annie and Clarabel

Most of the parts are common to both carriages, the only ones that differ are the sides.  For Annie you need 2 off the Annie side and for Clarabel you need one off right and one off left.

- Wheels.  These I resin printed but you could use Slaters.  You would have to make your own bearings or open out the bearing housings to ¼” and find suitable Imperial ball races.

- Axles; 3mm mild steel 65mm long.

- Bearings MR63ZZ.  These are available on E bay.  6mm OD, 3mm ID by 2.5mm wide.

- Axle boxes and W Irons. These I resin printed.  I opened out the bearing housings with a 6mm end mill.  The bearings were generally a press fit but if not a dab of super glue fixes them in place.

- Chassis, Filament printed.

- Sole bars; filament printed and glued to chassis.  I use No Nonsense super glue from ScrewFix, works well, is very versatile and is cheap!

- Sides, filament printed.  Best printed on end to reduce striations.

- Partitions are filament printed, note they go back to back but at the guard end there is a reduced one with no seats.  In Annie under the windows at the end there is no back.

- Roof, printed on end to reduce the striations.

- Decals, printed using a laser printer onto white waterslide.  I find it best to coat the decals with clear lacquer from Wilko before using them otherwise the lettering can be rubbed off when handling. I have uploaded these in Word so you can change the background colour if required.

- Hooks, Model Engineers Laser part number 24312 Gauge 1 Coupling Hooks.

- Coupling chain, 105 clock chain from various suppliers on E Bay. Wire (MIG wire) and spring from “will be useful one day box”.

- Buffer stocks, resin printed, buffer beams filament.

- Buffers turned from 10mm bar stock.

- Glazing, cut from old OHP slide material or Peter Wood can provide them laser cut in 1mm Perspex, see Model Engineers Name Plates.

- Paint; I used Painters Touch from RUST-OLEUM; Real Orange. This is the backing colour of the decals.

Annie

Clarabel