45’ GNR Bogie Coach Dia.129 and Dia.277 c. 1900

Peter F. Davis G1MRA #257 April 2026

From drawings in Nick Camplings book “Historic Carriage Drawings”.

These are my notes and some suggestions.

The book has drawings for three GNR coaches that use this common frame. I chose to model two of the three.

Nozzle .4mm.  Elegoo Neptune 3 Pro and Creality Ender 3 S1 filament printers used.

3/8” scale (9.5mm), printing with parts rotated 45 degrees on the build plate. This aligns the top layer with the panels.

I had good results using .1mm layers with .2mm initial layer. Printing most parts with 80 percent infill to add weight.

Body; Filament PLA “Overture” Matte Light Brown.

Roof, frame and trucks; black or gray PLA+, painting where necessary.

Purchased items and Some MSDS for Product used:

Cyanoacrylate Glue-Super Glue ( https://www.bsi-inc.com/sds_pdf/sds_insta_set.pdf )

Note of caution: Cyanoacrylate Glue (Super Glue). Young people and people unfamiliar with this product should seek knowledgeable adult supervision.

Un-Cure Cyanoacrylate Glue-Super Glue ( https://bsi-inc.com/sds_pdf/sds_un_cure.pdf ) Good to have on hand.

Permatex thread locker purple (https://www.permatex.com/wp-content/uploads/sds/24024.pdf )

Tamiya Putty white

G-S Hypo Cement.  A clear cement for watch crystals.

Screws: 2mm x 8mm long = 11

2mm x 12mm long = 6

Nuts:	2mm = 14

4 Pieces 3mm dia. Ground SS rod x 62mm long.

8 Pieces K&S Brass tube 4mm OD x .45mm Wall x 5mm long

Chapter 1 Trucks

Holes for screws drill #46 or .081.  Holes for 4mm bearings 5/32.

Wheels were drilled #32 or 3mm and assembled to axles with a bit of purple thread lock.

Assemble with screws snug, then back off screws just enough to leave the truck flexible.  Use purple thread lock.

(Should you choose, optional truck sides without step are given)

- 33.0 Wheel 		        	print 8

- 34.0 Truck side 	 		print 4

- 35.0 Bolster truck		print 2

- 36.0 End Brace			print 4

- 37.0 Truck retainer		print 2

- 38.0 Axle			print 4

Optional:

- 34.0 Truck side no step		print 4  		  See view in Chapter 6 for reference.

Chapter 2 Frame

- Half-frame were printed diagonally on the hot plate.  Assembled together with 2 bits of filament and screwed together.

- 41.0 Frame half 	print 2

- 42.0 Bolster frame	print 2

- 43.0 Truss rod		print 2

- 44.0 Step		print 2 or Omit

- 45.0 Gas Cylinders	print 1

- 46.0 Brake cylinder 	print 1

Chapter 3 Buffer/Coupler

The buffer and shank may have to be fitted. I used a small square diamond coated file to fit the shank to buffer then glued the head onto the shank with a drop of super glue.

Purple thread lock used to attach the assembled buffer to the buffer beam.

- 51.0  Hook			print 2

- 52.0  Hook Plate		print 2

- 53.1  Hook Spring		print 2

- 54.0  Buffer Head 		print 4

- 55.0  Buffer Head Shank   	print 4

- 56.0  Buffer 			print 4

Chapter 4 Body D129

The side/end pieces were printed with the inside surface down, rotated 45 degrees.  Floor pieces also rotated 45 degrees.

Window panes are .5mm thick, various widths and of 2 heights.  Held in place by individual frames.

- 11.0 Window Frames Lav. End 6 piece set 	print 2 sets

- 12.0 Window Frames Center 4 piece set		print 2 sets

- 13.0 Window Frames Lug. End 7 piece set	print 2 sets

Install the window panes and frames.

- Dry assemble the body on a flat surface. Inspecting as you go. Note the ends of the floor and sides are flush (as will be the roof).

- 8 printed clamps are used to hold the sides to the floor while gluing, 2 printed clips pull the two floor halves together while gluing.

- Use weights across the sides, holding the sides down to the floor piece. Move weights around to facilitate gluing if necessary.

- Note; the partitions are shorter than the 2 ends.

- Note of caution: Cyanoacrylate Glue (Super glue). Young people and people unfamiliar with this product should seek knowledgeable adult supervision.

- Gluing.  I started in the 4 corners with a drop of super glue running down to the floor. Next, the same at the 4 side seams and on to the partitions. Also a drop at the floor for each side and partition.

- Remove the two clips, install and glue the center partition.

- 3.0  Side 1		print 1

- 4.0  Side 2c		print 2

- 5.0  Side 3		print 1

- 6.0  Side 4		print 1

- 7.0  Side 5		print 1

- 8.0 End Lug.  		print 1

- 9.0 END Lav.		print 1

- 10.0  Partition		print 6

- 14.0  Floor Lav. end	Print 1

- 15.0 Floor Lug. End	print 1

- Assembly Clamp 72	print 8

- Assembly Clip 		print 2

- Assembly End Support	print 2

4a Install Seats D129

- 16.0 Seats First Class		print 2

- 17.0 Seats Third Class open	print 1

- 18.0 Seats third class		print 1

Chapter 5 Roof D129

- I printed the roof sections vertically.

- Trial fit the roof pieces. Slide the end roof section back and fourth over the body ends, correct any interference.

- File small groove on the sections where they join, this will allow glue to travel inside when glueing the roof sections together.

- With the 4 roof sections in position on top the coach, start to glue at one of these grooves and draw a bead up to the  top.  Once the glue has dried, turn the assembly upside down glue the roof to the ends

- I used “Tamiya Putty” to correct any mismatch at the roof joints. Putty, sand, paint until the finish is acceptable to you.

- Add the sky light, lamps and torpedo vents. Final coat of roof paint.

- 61.0 Roof Lug.  End			print 1

- 62.0 Roof Center Lug.  End		print 1

- 63.0 Roof Center Lav.  End		print 1

- 64.0 Roof Lav. End	 		Print 1

- 65.0 Sky light				print 1

- 66.0 Lamp				print 7

- 67.0 Torpedo Vent			print 16

Chapter 6 Extra

Now the thought came to me, if one were to make a second lavatory end, to replace the luggage end, they could have a “Lav.  All Third”.  This may or may not have happened in real life, but will add variety to your train. Hypothetically of course, featured briefly and shown with no step.

- 11.0 Window Frames Lav.  End-6 piece set 	print 4 sets

- 12.0  Window Frames Center-4 piece set	print 2 sets

- 3.0  Side 1		print 2

- 4.0  Side 2c		print 2

- 6.0  Side 4		print 2

- 8.0 END Lug.		print 1

- 9.0 END Lav.		print 1

- 10.0 Partition		print 6

- 14.0  Floor Lav. end	Print 2

- Assembly Clamp 72		print 9

- Assembly Clip 			print 2

- Assembly End Support		print 2

- 17.0 Seats Third Class open	print 2

- 18.0 Seats third class		print 2

- 63.0 Roof Center Lav.  End		print 2

- 64.0 Roof Lav.  End			Print 2

- 66.0 Lamp				print 8

- 67.0 Torpedo Vent			print 16

- Chapter 7  Brake 3rd  Dia. 277

As with D-129, the side/end pieces were printed with the inside surface down, rotated 45 degrees.  Floor pieces also printed, rotated 45 degrees.

For the sides; window panes are various widths and of 2 heights.  Held in place by individual frames.

The two panes for the end will require careful fitting. Here I used the watch crystal cement.

Panes are .5mm thick.

- Window frames Lav side 5-6,  6 piece set 	print 2 sets

- Window frames Center side  3-4,  2 piece set	print 2 sets

- Window frames Lookout-side 1-2,  3 piece set	print 2 sets

- No frames used for the two end panes.

Install the window panes and frames.

- Dry assemble the body on a flat surface. Inspecting as you go. Note the ends of the floor and sides are flush (as will be the roof).

- 7 printed clamps are used to hold the sides to the floor while gluing, 2 printed clips pull the two floor halves together while gluing. 2 printed end supports hold the sides in and support the floor from sagging under any added weight.

- Use weights across the sides, holding the sides down to the floor piece. Move weights around to facilitate gluing if necessary.

- Note; the partitions are shorter than the 2 ends.

- Note of caution: Cyanoacrylate Glue (Super glue). Young people and people unfamiliar with this product should seek knowledgeable adult supervision.

- Gluing.  I started in the 4 corners with a drop of super glue running down to the floor. Next, the same at the 4 side seams and on to the partitions. Also a drop at the floor for each side and partition.

- Remove the two clips, install and glue the center partition.

277-1 Side 1-Lookout left

277-2 Side 2-Lookout Right

277-3 Side 3-Center

277-4 Side 4-Center

277-5 Side 5 Lav right

277-6 Side 6 Lav Left

277-7 End 7-Lav

277-8 End 8-Lookout

277-9-Floor Lookout end

277-10-Floor Lav end

277-11A Partition	print 3

277-11B Partition	print 1

277-11C Partition	print 3

Assembly CLIP		print 2

Assembly clamp 72	print 7

Assembly 72 End support	print 2

7a Install Seats D277

277-12A Seats

277-12B Seats

277-12C Seats Lookout

Chapter 8 Roof D277

277-13 Roof-Lookout end

277-14 Roof-center Lookout end

277-15 Roof-Center lav. end

277-16 Roof-Lav. end

65.0 Skylight		print 3

Skylight Panes			 6 pieces 14.8mm x 6.9mm x .5mm clear

66.0 Lamp		print 7

67.0 Torpedo Vent	print 11

GN.200 Side Lamp	print 2

Exploded view D277

Peter F. Davis

G1MRA #257

17 April 2026